# Cadmium Biosensor Conditions Screening

13 October 2026

# Aim of the Experiment

The objective of this experiment was to test conditions for the cadmium biosensor (piGEM004?) in S. cerevisiae. Different concentrations of calcium and cadmium were added to the buffer, and changes in fluorescence were recorded. The cells were expected to show increased fluorescence after sensing Cd2+, which causes an influx of Ca2+ into the cell. The calcium then binds to the reporter, causing its fluorescence to increase.

Relevant literature:
cadmium response - https://febs.onlinelibrary.wiley.com/doi/10.1016/j.febslet.2014.07.001

GCaMP - Fast and sensitive GCaMP calcium indicators for imaging neural populations

GCaMP in Candida albicans - https://journals.asm.org/doi/10.1128/mbio.01157-23?url_ver=Z39.88-2003&rfr_id=ori:rid:crossref.org&rfr_dat=cr_pub%20%200pubmed

# Sample Preparation

Ruiqi brought the cells from INM and measured the OD immediately before transporting them.

All cultures were diluted to the same OD (see the beautiful image above for the volume of buffer added to achieve OD normalization).  (we can change this image into a table of course XD)

Ruiqi has the original data on OD.

500 uL of cells were centrifuged for 2 min at 4000 rpm to remove the media and were resuspended in the same volume of 60 mM PIPES buffer (pH 6.5). This was repeated three times.

100 uL of cells in buffer were added to the wells.

# Experimental Setup

96-well black flat-bottom plate (Greiner?).

Stock solutions of CaCl2 and CdCl2 were prepared at concentrations of 100 mM and 1 mM.

Images were acquired using the Nikon Axiovert confocal microscope.

# Conditions Tested and File Names

| Filename | Sample | Ca2+ (mM) | Cd2+ (mM) | Time | Well/Field | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| C1_0mM-calcium_0mM_cadmium_B5_1.nd2 | C1 | 0 | 0 | t0 | B5 | Baseline control |
| C1_0mM-calcium_0mM-cadmium_B6_t0min_1001.nd2 | C1 | 0 | 0 | t0 | B6 | Baseline control |
| C1_1mM-calcium_0mM_cadmium_B5_t0_1001.nd2 | C1 | 1 | 0 | t0 | B5 | Calcium only |
| C1_1mM-calcium_0mM_cadmium_B5_t0_1.nd2 | C1 | 1 | 0 | t0 | B5 | Calcium only (replicate acquisition) |
| C1_1mM-calcium_0mM_cadmium_B5_t5min_1.nd2 | C1 | 1 | 0 | 5 min | B5 | Calcium only |
| C1_10mM-calcium_5mM-cadmium_B6_t5min_1001.nd2 | C1 | 10 | 5 | 5 min | B6 | High Ca + high Cd |
| C2_0mM-calcium_0mM_cadmium_C5_1.nd2 | C2 | 0 | 0 | t0 | C5 | Baseline control |
| C2_1mM-calcium_0mM_cadmium_C5_t0_1.nd2 | C2 | 1 | 0 | t0 | C5 | Calcium only |
| C2_1mM-calcium_0uM-cadmium_B6_t0min_1.nd2 | C2 | 1 | 0 | t0 | B6 | Calcium only |
| C2_10mM-calcium_500uM-cadmium_B5_t5min_1.nd2 | C2 | 10 | 0.5 | 5 min | B5 | High Ca + low Cd |
| C2_1mM-calcium_5mM-cadmium_B6_t5min_2.nd2 | C2 | 1 | 5 | 5 min | B6 | Low Ca + high Cd |
| C2_10mM-calcium_5mM-cadmium_B7_t5min_1001.nd2 | C2 | 10 | 5 | 5 min | B7 | High Ca + high Cd |
| C6_0mM-calcium_0mM_cadmium_D5_1.nd2 | C6 | 0 | 0 | t0 | D5 | Baseline control |
| C6_10mM-calcium_500uM-cadmium_D7_t5min_1001.nd2 | C6 | 10 | 0.5 | 5 min | D7 | High Ca + low Cd |
| C6_10mM-calcium_5mM-cadmium_D6_t5min_1001.nd2 | C6 | 10 | 5 | 5 min | D6 | High Ca + high Cd |
| C8_0mM-calcium_0mM_cadmium_E5_1.nd2 | C8 | 0 | 0 | t0 | E5 | Baseline control |
| C8_10mM-calcium_5mM-cadmium_E6_t5min_1001.nd2 | C8 | 10 | 5 | 5 min | E6 | High Ca + high Cd |
| Empty_0mM-calcium_0mM_cadmium_A5_1.nd2 | Empty | 0 | 0 | t0 | A5 | Negative control |
| Empty_0mM-calcium_0mM-cadmium_A6_t0min_1001.nd2 | Empty | 0 | 0 | t0 | A6 | Negative control |
| Empty_10mM-calcium_5mM-cadmium_A6_t5min_1001.nd2 | Empty | 10 | 5 | 5 min | A6 | Negative control with ions |

# Observations

A substantial number of colonies exhibited increased fluorescence. Quantification need to be done, but I am unfamiliar with that and Naresh will help out soon.

## Response at 5 mM Cd2+

This condition produced the strongest response, and multiple examples of sensor activation were observed. The Cd2+ concentration of 5 mM is high, but gave clear responses in around 2 minutes after addition to the well.

## Response at 500 µM Cd2+

The 500 µM Cd2+ concentration was less effective. however, I was able to capture some cells flaring up in response to cadmium.

In the case shown, the fluorescence increase was not due to cells changing position and entering the focal plane (this is true for the other images above as well).

## Effect of Calcium Concentration

Increased fluorescence was observed at both 1 mM and 10 mM Ca2+ concentrations, but did not seem to correlate with the amount of calcium added:

## Empty-Vector Control

The empty vector control showed no detectable change in fluorescence over the course of the timelapse experiment. Faint autofluorescence was barely visible.

(The small green dots visible in the image are imaging noise.)

# Fluorescence Quantification

Needs to be done! @Nareshyboy #mastrmicroscopist

# Preliminary Conclusions

Cells carrying the cadmium biosensor exhibited increased fluorescence following exposure to Cd2+, with the strongest responses observed at 5 mM Cd2+. Fluorescence activation was observed at both 1 mM and 10 mM Ca2+ concentrations. In contrast, the empty-vector control showed no detectable increase in fluorescence over time. These observations suggested that the biosensor responded to cadmium exposure under multiple calcium concentrations and that 5 mM Cd2+ produced the most pronounced response among the conditions tested.